# TASK-2

TITLE: Develop a simple synchronous RAM module with read and write operations

OBJECTIVE: To design and simulate a simple synchronous RAM using Verilog HDL that performs read and write operations based on clock pulse

SOFTWARE USED:  vivado

THEORY: RAM (Random Access Memory) is a memory device used to store data temporarily. In synchronous RAM, all operations happen only at the active edge of the clock signal.

Operations:

Write Operation: Data is stored in memory when Write Enable (we) = 1.

Read Operation: Stored data is sent to output when Write Enable (we) = 0.

Features:

16 memory locations

Each location stores 8-bit data

Controlled by clock signal

CODE:

VERILOG CODE FOR RAM:

`timescale 1ns / 1ps

///ram design

module task_2(

input clk,

input we,                 // Write Enable

input [3:0] addr,         // 16 locations

input [7:0] din,          // Input Data

output reg [7:0] dout     // Output Data

):

reg [7:0] mem [15:0];

always @(posedge clk)

begin

if(we)

mem[addr] <= din;     // Write operation

else

dout <= mem[addr];    // Read operation

end

endmodule

TESTBENCH FOR THAT VERILOG CODE:

`timescale 1ns / 1ps ///testbench for ram design

module task_2_tb;

reg clk;

reg we;

reg [3:0] addr;

reg [7:0] din;

wire [7:0] dout;

task_2 uut(

.clk(clk),

.we(we),

.addr(addr),

.din(din),

.dout(dout));

always #5 clk = ~clk;

initial begin

clk = 0;

// Write 55 at address 2

we = 1; addr = 4'd2; din = 8'd55;

#10;

// Write 99 at address 5

we = 1; addr = 4'd5; din = 8'd99;

#10;

// Read address 2

we = 0; addr = 4'd2;

#10;

// Read address 5

we = 0; addr = 4'd5;

#10;

$stop;

end

endmodule

SIMULATION RESULT:

Write cycles:

Data 37 written into address 2

Data 63 written into address 5

Read cycles:

Address 2 output = 37

Address 5 output = 63

The waveform confirms successful read and write operations

TRUTH TABLE:

| We | Operation |
| --- | --- |
| 1 | Write |
| 0 | Read |

OUTPUT OBSERVATION TABLE:

| Time(ns) | we | Address | Data in | Data out |
| --- | --- | --- | --- | --- |
| 10 | 1 | 2 | 37 | XX |
| 20 | 1 | 5 | 63 | XX |
| 30 | 0 | 2 | -- | 37 |
| 40 | 0 | 5 | -- | 63 |

PRACTICAL OUTPUT:

CONCLUSION:

A synchronous RAM module was successfully designed and simulated using Verilog HDL. The RAM correctly stored input data during write mode and produced stored data during read mode.